Примерна програма за процесор PowerPC

Под - програма за намиране на средно аритметична стойност на масив, зададен с начален адрес в регистър “r3” и брой елементи равни на 2 на степен числото зададено с променливата POWER. Резултатът да се получи в регистър „r3”.

#elif defined __powerpc__

li		%r4,0			; нулира регистър r4

; за младша част на сумата

; load Immediate => r4 <- 0

li 		%r5,0			; нулира регистър r5

; за старша част на сумата

; load Immediate => r5 <- 0

li		%r0,1 << POWER	; изчислява размер на масива

; брой елементи = 2 на степен POWER

; r3 <- „1: изместена POWER пъти наляво

; load Immediate => r0 <- 1<< power

mtctr		%r0			; записва намерената стойност в регистър

; ctr,който ще служи за брояч на циклите

;ctr <- r0

addi 		%r3,%r3,-4		; намира адреса преди началото на масива

; load Immediate => r3 <- r3 -4

L1:	 lwzu 		%r0,4(%r3)		; зарежда в регистър “r0” текущя елемент

; от масива регистър “r3” съдържа адреса на

; предния елемент от масива. Преди четене от

; масива, адреса в “r3” нараства с 4

; load word and zero with update

; r0 <- EA=r3+4 ; r3 <- EA

addc 		%r4,%r4,%r0		; събира натрупаното с текущя елемен

; r4 <- [r4] + [r0]

addze 	%r5,%r5		; получава старши разряд на сумата

; Add to Zero Extended => r5 <- r5 + XER[CA]

bdnz		L1			; намалява брояча на циклите и

; ако не е 0 – правим преход към

; началото на цикъла

; Decrement CTR, branch if CTR ≠ 0

rlwinm		%r3,%r4,32 - POWER,POWER,31

; дели на брой елементи

; Rotate Left Word Immediate then AND with

; Mask

;rlwinm	rA,rS,SH,MB,ME

rlwimi		%r3,%r5,32 - POWER,0,POWER – 1

; ; получава старши разряд на резултата

blr					; край на под-програмата

Описание на по-важните команди, които са използвани в програмата

li - Load Immediate

The addi and addis instructions can be used to load an immediate value into a register. Additional mnemonics are provided to convey the idea that no addition is being performed but that data is being moved from the immediate operand of the instruction to a register.

- Load a 16-bit signed immediate value into rD.

li 	rD,value 		equivalent to 		addi rD,0,value

- Load a 16-bit signed immediate value, shifted left by 16 bits, into rD.

lis 	 rD,value 		equivalent to 		addis rD,0,value

mtctr -  Copy the contents of rS to the CTR

mtctr rS 		equivalent to 		mtspr 9,rS

lwzu - Load Word and Zero with Update ()

addcx - Add Carrying

Addc	rD, rA, rB	; rD <- (rA) + (rB)

adze – Add to Zero Extended

bdnz - Decrement CTR, branch if CTR ≠0

bdnz target 		equivalent to 		bc 16,0,target

rlwinm rA,rS,n,b – n,31 – n Clear left and shift left word immediate

rlwimi rA,rS,32 – b,b,(b + n) – 1 Insert from left word immediate